# How Correlation is Calculated in fcs_corr.py

A detailed technical reference connecting the code to the underlying mathematics of fluorescence correlation spectroscopy.

## 1. What G(τ) measures

Fluorescence correlation spectroscopy extracts information about molecular dynamics by measuring how the fluorescence intensity at time t is related to the intensity at time t + τ. The normalised correlation function is defined as:

G(τ) = ⟨δI(t) · δI(t + τ)⟩ / ⟨I(t)⟩²

where δI(t) = I(t) − ⟨I(t)⟩ is the fluctuation around the mean intensity. For an ideal freely diffusing dye the result is a curve that starts above zero at short lags (photons are correlated because they came from the same molecule) and decays to zero at long lags (the molecule has left the focal volume and the two time points are uncorrelated). The lag at which G(τ) = G(0)/2 is related to the diffusion time through the focal volume.

For the cross-correlation between two detection channels A and B the definition generalises to:

G_AB(τ) = ⟨δI_A(t) · δI_B(t + τ)⟩ / (⟨I_A⟩ · ⟨I_B⟩)

The autocorrelation is the special case where A = B — both inputs are the same photon stream.

## 2. The data representation: a list of arrival times

FCS experiments record individual photon events in TTTR (time-tagged time-resolved) mode. After reading the binary file, FCSData stores each channel as a sorted array of absolute photon arrival times in seconds:

# fcs_reader.py — FCSData

@property

def ch1_times_s(self) -> np.ndarray:

"""Absolute photon arrival times for Ch1, in seconds from t=0."""

return np.cumsum(self.ch1_deltas.astype(np.float64)) * self.macrotime_period_s

The binary file stores differences between consecutive macrotimes (delta encoding), so a cumulative sum reconstructs the absolute timeline. Dividing by macrotime_period_s (= 1 / clock_Hz) converts clock cycles to seconds. The result is a 1-D float64 array of length N, one entry per detected photon, in strictly increasing order.

This representation — a sparse list of event times rather than a binned intensity trace — is the natural input for a time-tagged correlator and avoids any loss of time resolution from pre-binning.

## 3. Connecting the integral to countable pairs

Starting from the continuous definition, the unnormalised correlation at lag τ can be written as a sum over all ordered pairs of photons:

C(τ) = Σ_i Σ_j δ(t_j^B − t_i^A − τ)

Integrating over a finite lag bin [τ_k, τ_{k+1}) converts the Dirac delta into a count:

C_k = number of pairs (i, j) such that τ_k ≤ t_j^B − t_i^A < τ_{k+1}

This is the exact quantity that every backend in fcs_corr.py computes. There are no approximations at this stage — every photon pair is either counted or not.

## 4. Building the lag axis — build_tau_edges

FCS curves span many decades of lag time (microseconds to seconds), so the lag axis is always log-spaced. The function build_tau_edges constructs the bin edge array:

# fcs_corr.py

_POINTS_PER_DECADE = 30

def build_tau_edges(tau_min_s: float, tau_max_s: float) -> np.ndarray:

log_min = np.log10(tau_min_s)

log_max = np.log10(tau_max_s)

n = max(10, int(round((log_max - log_min) * _POINTS_PER_DECADE)) + 1)

return np.logspace(log_min, log_max, n)

With 30 points per decade a [0.01 ms, 1000 ms] range (5 decades) yields 151 edges → 150 bins. Log spacing is essential for two reasons:

- Biological relevance. Diffusion times of interest range from ~10 µs (fast dyes) to ~100 ms (slow complexes), spanning four or more decades. A linear axis would crush the short-lag behaviour completely.

- Counting statistics. At long lags, the number of photon pairs per bin grows with the bin width Δτ_k. Log spacing keeps the fractional bin width Δτ_k / τ_k approximately constant, so each bin contains a similar number of pairs and carries comparable statistical weight.

The returned array is passed unchanged as tau_edges through all subsequent functions.

## 5. Computing the pair counts — the two backends

### 5.1  Per-bin searchsorted (_correlate_perbin)

This is the default backend. Its key insight is that for a fixed lag bin k, finding all pairs is equivalent to: for every A photon at time t_A, counting B photons in the interval [t_A + τ_k, t_A + τ_{k+1}). Because timesB is sorted, both boundaries can be found with binary search simultaneously for all A photons using numpy.searchsorted:

# fcs_corr.py — _correlate_perbin

for k in range(nBins):

lo = np.searchsorted(timesB, timesA + tau_edges[k],     side='left')

hi = np.searchsorted(timesB, timesA + tau_edges[k + 1], side='left')

counts[k] = float(np.sum(hi - lo))

The expression timesA + tau_edges[k] is a NumPy broadcast: it shifts the entire A array by a scalar lag edge in one operation, producing a length-N array of query points. searchsorted then finds the insertion point for each query in timesB — effectively a vectorised binary search. The number of B photons in the window for A photon i is hi[i] - lo[i], and summing over all i gives C_k.

There is no Python loop over photons — only a Python loop over lag bins (at most ~150 iterations). The computational work per bin is O(N log N) for the two searchsorted calls, giving an overall complexity of O(n_bins × N log N).

### 5.2  Wahl two-pointer (_correlate_twopointer_numpy and _correlate_twopointer_numba)

This is a direct translation of the MATLAB tcspc_crosscorr_directed.m algorithm due to Wahl et al. Its structure is a nested scan rather than a per-bin vectorisation:

# fcs_corr.py — _correlate_twopointer_numpy

j_start = 0

for i in range(len(timesA)):

tA = timesA[i]

# Advance j_start past photons too early to contribute

while j_start < NB and timesB[j_start] < tA + minTau:

j_start += 1

# Walk forward through B photons in the lag window

j = j_start

while j < NB:

dt = timesB[j] - tA

if dt > maxTau:

break

idx = np.searchsorted(tau_edges, dt, side='right') - 1

counts[idx] += 1.0

j += 1

The outer loop walks through every A photon. For each one, j_start is advanced past any B photons that are too early (lag < τ_min). This advancement is permanent — j_start only moves forward — because both arrays are sorted: if a B photon is too early for A photon i, it is also too early for all subsequent A photons. Then a temporary pointer j scans forward from j_start, computing each lag dt = timesB[j] − tA and using a binary search to assign it to the correct bin.

The key difference from the perbin approach is directionality: the pair is found by walking B for each A, rather than by querying all A photons simultaneously for each lag bin. For datasets with many photons at similar rates this makes the total work O(N_A + N_B) for pointer advancement plus O(P · log n_bins) for the pair assignments, where P is the number of pairs inside [τ_min, τ_max]. In the worst case P = O(N²).

The _numba variant replaces np.searchsorted (unavailable in Numba's no-Python mode) with an inlined binary search:

# fcs_corr.py — _correlate_twopointer_numba

left, right = 0, nBins - 1

while left <= right:

mid = (left + right) >> 1

if dt < tau_edges[mid]:

right = mid - 1

elif dt >= tau_edges[mid + 1]:

left = mid + 1

else:

counts[mid] += 1.0

break

The >> 1 is a bitwise right-shift, equivalent to integer division by 2 but faster in compiled code. The search terminates when tau_edges[mid] ≤ dt < tau_edges[mid + 1], placing dt unambiguously in bin mid.

### 5.3  Backend dispatch

The private _correlate function selects the backend, including the logic for switching to Numba above a photon-count threshold:

# fcs_corr.py — _correlate

if method == 'twopointer':

N = max(len(timesA), len(timesB))

if _NUMBA and N >= _NUMBA_THRESHOLD:

return _correlate_twopointer_numba(timesA, timesB, tau_edges)

return _correlate_twopointer_numpy(timesA, timesB, tau_edges)

_NUMBA_THRESHOLD = 50_000. Below that threshold the overhead of JIT compilation and array marshalling into Numba's memory model can exceed the speedup gained, so the pure-Python path is preferred.

## 6. Normalising to G(τ) — _normalize

Raw pair counts C_k cannot be compared across experiments or even across lag bins, because they scale with: the total measurement time (longer experiments accumulate more pairs), the photon count rates (brighter samples produce more pairs), and the lag bin width (wider bins catch more pairs). The function _normalize applies the standard FCS normalisation:

# fcs_corr.py — _normalize

tau        = 0.5 * (tau_edges[:-1] + tau_edges[1:])

bin_widths = np.diff(tau_edges)

T          = (max(timesA[-1], timesB[-1])

- min(timesA[0],  timesB[0]))

rateA = len(timesA) / T

rateB = len(timesB) / T

norm  = rateA * rateB * (T - tau) * bin_widths

G = np.where(norm > 0, counts / norm - 1.0, np.nan)

Each line corresponds to a term in the normalisation formula:

G(τ_k) = C_k / [ r_A · r_B · (T − τ_k) · Δτ_k ] − 1

tau — bin centres. The midpoint 0.5 * (τ_k + τ_{k+1}) is the representative lag for bin k, used for both the (T − τ) correction and as the x-axis value in plots.

bin_widths — Δτ_k. np.diff(tau_edges) is the width of each lag bin. On a log axis these grow geometrically, which is why they must appear explicitly: without this factor, short bins would appear under-counted relative to wide bins.

T — the observation window. This is the overlap duration of the two photon streams. For autocorrelation (same stream passed twice), T is simply the measurement duration. For cross-correlation between two channels that may start or end at slightly different times, the intersection [max(t0_A, t0_B), min(t_end_A, t_end_B)] is used.

(T − τ_k) — the lag correction. For any given lag τ_k, only A photons that arrive before t_end − τ_k can in principle have a B partner at lag τ_k. The effective window for pair formation is T − τ_k rather than T. Without this correction, long-lag bins would appear systematically depressed because they are available to fewer A photons. This term is essential for an unbiased estimate of G(τ) at lags that are an appreciable fraction of T.

rateA · rateB converts the count-based denominator into the same units as the numerator. rateA = N_A / T is the mean photon rate on channel A in Hz. The product r_A · r_B · (T − τ) · Δτ is the number of pair events you would expect from two uncorrelated Poisson processes with the same rates. Dividing C_k by this expected count gives 1 for uncorrelated data, so subtracting 1 shifts the baseline to 0.

NaN guard. The np.where(norm > 0, ...) protects against zero or negative denominators that could arise from very sparse data or extreme lags.

## 7. Autocorrelation as a special case

The public API separates cross- and auto-correlation, but internally they share a single implementation:

# fcs_corr.py — compute_autocorr

def compute_autocorr(

times_s: np.ndarray,

tau_edges: np.ndarray,

method: Method = 'perbin',

segment: bool = False,

) -> Tuple[np.ndarray, np.ndarray, np.ndarray, int]:

return compute_segmented(times_s, times_s, tau_edges, method, segment)

Passing times_s as both A and B means every photon is correlated against every other photon in the same stream, including itself. The self-pair (i = j, lag = 0) never contributes because τ = 0 is not inside any lag bin: the lag axis starts at τ_min > 0. At τ → 0 the autocorrelation amplitude G(0) is related to the number of molecules in the focal volume: G(0) = 1/N for N identical freely-diffusing molecules.

Because the same array is used twice, the cross-correlation code path computes both "A before B" and "B before A" pairs — but since A and B are identical streams these are exactly the same pairs, so C_k counts each pair once from each direction, giving the correct symmetric autocorrelation.

## 8. Segmentation and uncertainty — compute_segmented

The main computation function compute_segmented has two operating modes controlled by the segment flag.

### 8.1  Full-dataset mode (default)

# fcs_corr.py — compute_segmented, segment=False path

t_start = max(timesA_s[0], timesB_s[0])

t_end   = min(timesA_s[-1], timesB_s[-1])

maskA   = (timesA_s >= t_start) & (timesA_s <= t_end)

maskB   = (timesB_s >= t_start) & (timesB_s <= t_end)

segA    = timesA_s[maskA] - t_start

segB    = timesB_s[maskB] - t_start

counts  = _correlate(segA, segB, tau_edges, method)

G_mean  = _normalize(counts, segA, segB, tau_edges)

G_std   = np.full_like(G_mean, np.nan)

return tau, G_mean, G_std, 1

The streams are trimmed to their overlap window and re-zeroed (subtracting t_start) before being passed to the correlator. Re-zeroing is required because _normalize measures T from the first to the last photon in the supplied array — without it, T would be artificially large if the first photon arrives long after time zero.

G_std is filled with NaN to signal that no uncertainty estimate is available. The plotting function checks np.all(np.isnan(G_std)) and omits the grey ±1σ bands when this is the case.

### 8.2  Segmented mode

When segment=True, the total measurement is divided into non-overlapping chunks and each is correlated independently:

# fcs_corr.py — compute_segmented, segment=True path

seg_duration_s = _MIN_SEGMENT_FACTOR * tau_max

for k in range(n_segs):

lo = t_start + k * seg_duration_s

hi = lo + seg_duration_s

maskA = (timesA_s >= lo) & (timesA_s < hi)

maskB = (timesB_s >= lo) & (timesB_s < hi)

segA  = timesA_s[maskA] - lo

segB  = timesB_s[maskB] - lo

counts = _correlate(segA, segB, tau_edges, method)

G_segments.append(_normalize(counts, segA, segB, tau_edges))

The minimum segment duration is set to _MIN_SEGMENT_FACTOR × τ_max, where _MIN_SEGMENT_FACTOR = 10. This ensures that within any single segment, the longest lag bin still has a reasonably large effective window T − τ_max = 0.9 T, preventing the lag correction from inflating uncertainty at long lags. Shorter segments would mean that the lag correction itself accounts for a significant fraction of T, making the normalised values unreliable.

The segment results are stacked and reduced:

# fcs_corr.py

G_stack = np.array(G_segments)

G_mean  = np.nanmean(G_stack, axis=0)

G_std   = np.nanstd(G_stack, axis=0, ddof=1) if n_valid > 1 \

else np.full_like(G_mean, np.nan)

nanmean and nanstd ignore NaN values that appear in individual bins (for example, a sparse segment may have no pairs in a particular lag bin). ddof=1 applies Bessel's correction, giving an unbiased estimate of the standard deviation across segments. A warning is raised when fewer than _MIN_SEGMENTS = 5 segments are available, since the standard deviation of fewer than five samples is a poor estimator.

## 9. The complete pipeline

Tracing a single call from the GUI to the final plot:

run_correlation_dialog(fcs_data)

│

├─ fcs_data.ch1_times_s          ← cumsum(deltas) / clock_hz

├─ fcs_data.ch2_times_s          ← cumsum(deltas) / clock_hz

│   [optional: apply_time_gate filters by microtime]

│

├─ build_tau_edges(τ_min, τ_max) ← 30 log-spaced edges per decade

│

├─ compute_crosscorr / compute_autocorr

│       └─ compute_segmented

│               ├─ clip & re-zero the photon streams

│               └─ for each segment (or once for full dataset):

│                       ├─ _correlate(segA, segB, tau_edges, method)

│                       │       └─ _correlate_perbin   ← default

│                       │          (or _twopointer_numpy / _numba)

│                       │          returns C_k: raw pair counts per bin

│                       └─ _normalize(counts, segA, segB, tau_edges)

│                               └─ G_k = C_k / (r_A·r_B·(T−τ_k)·Δτ_k) − 1

│

└─ plot_correlation(tau, G_mean, G_std, ...)

The separation between _correlate (returns C_k) and _normalize (converts C_k to G(τ)) is architecturally important: it makes it straightforward to add new backends (e.g. the planned Wiener–Khinchin FFT correlator) without touching the normalisation logic, and to test each stage independently.

## 10. Key constants and their physical justification

| Constant | Value | Role |
| --- | --- | --- |
| _POINTS_PER_DECADE | 30 | ~4% fractional bin width; resolves FCS features without excessive computation |
| _MIN_SEGMENT_FACTOR | 10 | Ensures T − τ_max ≥ 0.9 T; keeps long-lag bins well-sampled within each segment |
| _MIN_SEGMENTS | 5 | Minimum segments for a statistically meaningful standard deviation |
| _NUMBA_THRESHOLD | 50,000 | Break-even photon count above which JIT compilation pays off for the two-pointer |

## 11. Bugs and limitations to be aware of

to_dataframe typo in fcs_reader.py. The method references I2 (which is not defined in that scope) instead of I1. This will raise a NameError if called. The fix is to replace I2 with I1 in the return statement on that line.

_normalize uses last/first photon times, not segment boundaries. For the segmented path, each segment is re-zeroed before being passed to _normalize, so T is computed from the re-zeroed photon times. If a segment happens to have no photons for a brief interval at its start or end (a photon count dropout), T will be slightly underestimated, inflating G slightly for that segment.

Autocorrelation pair counting direction. Because timesA and timesB are the same array, the two-pointer backend counts each true pair twice (once with A as "reference" photon and B as "delayed", once with B as "reference" and A as "delayed"). The normalisation denominator is r_A · r_B = r², which also doubles the expected count, so the two factors cancel and the result is correct. The perbin backend has the same behaviour by symmetry.

No shot-noise correction. At τ = 0, detected photons are correlated with themselves (a single detection event cannot produce a pair at exactly zero lag, but practical bins at very short lags can still be contaminated by detector after-pulsing). The code does not apply a Koppel-style shot-noise correction; short-lag bins should be interpreted cautiously.